# Airfoil Dataset Dictionary

This document provides simple definitions of the variables used in the ML Powered Airfoil OE Project .The variables describe airfoil identity, aerodynamic conditions, performance measures, shape summary features, and coordinate-based geometry.

# Column Definitions:

## airfoil_id

This is the name or identifier of the airfoil design. It tells us which specific airfoil the row belongs to, such as NACA2412 or Clark-Y.

## reynolds

Reynolds number describes the flow condition of air around the airfoil. It depends on airspeed, density, size, and viscosity, and strongly affects lift and drag.

## aoa

AoA stands for Angle of Attack, which is the angle between the incoming airflow and the airfoil chord line. Increasing AoA usually increases lift up to a limit, after which stall may occur.

## cl

Cl is the Lift Coefficient, which measures how effectively the airfoil generates lift. Higher Cl values generally mean greater lifting capability.

## cd

Cd is the Drag Coefficient, which measures aerodynamic resistance experienced by the airfoil. Lower Cd values indicate less drag and better efficiency.

## cm

Cm is the Moment Coefficient or pitching moment coefficient. It indicates whether the airfoil creates a nose-up or nose-down rotational tendency during flight.

## cl_cd_ratio

This is the Lift-to-Drag ratio (Cl/Cd), which measures aerodynamic efficiency. A higher value means the airfoil produces more lift for a given amount of drag.

## max_thickness

Maximum thickness represents the largest distance between the upper and lower airfoil surfaces. It affects structural strength, drag, and aerodynamic behavior.

## thickness_location

Thickness location shows where along the chord the airfoil reaches maximum thickness. The value is normalized from 0 (front edge) to 1 (rear edge).

## max_camber

Maximum camber measures the greatest curvature of the airfoil’s camber line. More camber generally increases lift but may also increase drag.

## camber_location

Camber location indicates where along the chord maximum camber occurs. This affects pressure distribution, lift generation, and pitching moment.

## xu_1 … xu_n

These are the x-coordinates of points on the upper surface of the airfoil. They indicate horizontal position along the chord.

## yu_1 … yu_n

These are the y-coordinates of points on the upper surface. They describe the height and shape of the upper airfoil curve.

## xl_1 … xl_n

These are the x-coordinates of points on the lower surface of the airfoil. Like xu, they show horizontal location along the chord.

## yl_1 … yl_n

These are the y-coordinates of points on the lower surface of the airfoil. They define the lower curve and complete the airfoil shape.

# Dataset Structure Summary

| Column Type | Purpose |
| --- | --- |
| airfoil_id | Airfoil identity |
| reynolds, aoa | Flow conditions |
| cl, cd, cm, cl_cd_ratio | Aerodynamic performance |
| max_thickness, thickness_location, max_camber, camber_location | Shape summary features |
| xu/yu/xl/yl | Full airfoil geometry |